generate the exact byte and ID values for a specific VESC CAN command For rpm value of 2000

To send a VESC CAN command to set an ERPM (electrical RPM) value of 2000 using the traffic generator with the correct CAN ID and data payload, here is how to construct the exact CAN message:

VESC CAN Set RPM Command Format

- The VESC command for setting RPM uses the extended 29-bit CAN ID format:CAN ID=(Command ID<<8)|VESC device ID

- Command ID for "Set RPM" is 3 (0x03).

- VESC device ID is usually 0x01 (or whatever the target ID is).

- For example, with device ID 0x01:CAN ID=(0x03<<8)|0x01=0x0301

- As a 29-bit extended ID, this would be placed in the extended ID field.

Data Payload for Set RPM

- The RPM value is sent as a signed 32-bit integer (int32_t).

- The value is in ERPM units, so for 2000 RPM:data=2000 encoded as a 4-byte little-endian int32

- Remaining bytes in CAN frame data can be zeros if data length is 8 bytes.

Example Bytes for RPM = 2000:

- RPM 2000 decimal is 0x000007D0 in hex (32-bit signed int).

- Data byte array (8 bytes, little endian):[0xD0,0x07,0x00,0x00,0x00,0x00,0x00,0x00]

Summary

| Field | Value |
| --- | --- |
| CAN ID (29-bit) | 0x00000301 (Set RPM command + device ID 1) |
| Extended ID | Yes |
| Data Length | 8 bytes |
| Data bytes | D0 07 00 00 00 00 00 00 (rpm 2000 LE 32-bit int) |

You can input this in Kvaser CanKing traffic generator as:

- Extended ID: 0x00000301

- Data: D0 07 00 00 00 00 00 00

- DLC: 8

If your VESC device ID differs, replace the low byte of the ID accordingly.

⁂

- https://vesc-project.com/sites/default/files/imce/u15301/VESC6_CAN_CommandsTelemetry.pdf

- https://vesc-project.com/node/3523

- https://dongilc.gitbook.io/openrobot-inc/tutorials/control-with-can

- https://www.youtube.com/watch?v=QM5_vCy_uWk

- https://foil.zone/t/vesc-tool-thread-how-to-program/9670?page=2

- https://forum.esk8.news/t/optimum-vesc-settings/4884?page=3

- https://supzero.ch/vesc_config/params.php

- https://endless-sphere.com/sphere/threads/vesc-custom-applications.72062/

- https://endless-sphere.com/sphere/threads/any-controllers-capable-of-maintaining-smooth-rpm-decay-rate-under-suddenly-varying-load.122324/

- https://pev.dev/t/vesctool-configuration-guide/2317

- https://foil.zone/t/question-for-other-using-vesc-regarding-motor-rpm/16339

- https://vesc-project.com/node/659

- http://vedder.se/2015/01/vesc-open-source-esc/comment-page-1/

- https://www.reddit.com/r/VESC/comments/1d9yrq9/how_to_set_rpm_in_ui/

- https://www.vesc-project.com/node/1102

- https://www.vesc-project.com/node/3523

- https://www.scribd.com/document/547487406/VESC6-CAN-CommandsTelemetry

- https://f1tenth.readthedocs.io/en/stable/getting_started/firmware/firmware_vesc.html

- https://endless-sphere.com/sphere/threads/vesc-uart-communication.73325/